# PCB DESIGN & MANUFACTURING DOCUMENT

## RC CAR CONTROL SYSTEM PCB

## 1. Scope & Objective

### 1.1 Scope

The scope of this project is to design and manufacture a compact, reliable, and manufacturable PCB for an RC (Remote Controlled) Car, capable of:

- Receiving wireless control signals

- Driving DC motors for propulsion

- Controlling a servo motor for steering

- Managing battery power safely and efficiently

The PCB integrates control, power, and communication subsystems on a single board.

### 1.2 Objective

- Design a 2-layer PCB suitable for low-cost manufacturing

- Support high-current motor paths without overheating

- Ensure stable MCU and RF operation in a noisy motor environment

- Follow industry-standard PCB design rules

- Generate production-ready fabrication and assembly files

## 2. System Schematic Design

### 2.1 Functional Blocks

- Power Input & Protection

- Voltage Regulation

- Microcontroller Unit (MCU)

- Motor Driver Circuit

- Servo Interface

- Wireless Communication Module

- Programming & Debug Interface

### 2.2 Schematic Description

#### Power Section

- Battery Input: 7.4V Li-ion/LiPo

- Reverse polarity protection (P-channel MOSFET / diode)

- Bulk capacitor (470µF–1000µF) near battery

- Buck converter: 7.4V → 5V

- LDO: 5V → 3.3V (RF module)

Important Point:
Motor power and logic power must be filtered and decoupled separately.

#### MCU Section

- Example MCU: ATmega328P / STM32 / ESP32

- Decoupling capacitors (0.1µF) near each VCC pin

- Reset pull-up resistor (10kΩ)

- Programming header (ISP / SWD / UART)

#### Motor Driver Section

- Dual H-Bridge motor driver (TB6612FNG / L298)

- Flyback diodes (if required)

- PWM control from MCU

- High-current motor connectors

Important Point:
Motor driver ground should connect to system ground at a single star point.

#### Servo Control

- Standard 3-pin servo header (VCC, GND, PWM)

- Separate power trace from logic supply

#### RF Communication

- Module: nRF24L01 / HC-12 / ESP32

- 3.3V regulated supply

- SPI/UART communication

- Antenna clearance area

## 3. PCB Layout – Placement & Routing

### 3.1 Component Placement

| Area | Placement Rule |
| --- | --- |
| MCU | Center of board |
| Motor Driver | Near motor connectors |
| Buck Converter | Near battery input |
| RF Module | At PCB edge |
| Programming Header | Board edge |

Important Points:

- Keep RF module away from motor driver

- Short, direct paths for high-current components

- Place decoupling capacitors as close as possible to IC pins

### 3.2 Routing Guidelines

#### Signal Routing

- Use short traces for clock and RF signals

- Avoid routing signal traces under motor driver

- Maintain 45° trace angles

#### Power Routing

- Motor traces ≥ 80–120 mil (depending on current)

- Use copper pours for power and ground

- Bottom layer: solid ground plane

Important Point:
Never route motor current under MCU or RF module.

## 4. Footprint Assignment & PCB Design Rules (DRC)

### 4.1 Footprint Assignment

- All footprints must match manufacturer datasheets

- Use through-hole connectors for motors and battery

- Use SMD components for compact layout

### 4.2 PCB Design Rules (Typical)

| Parameter | Value |
| --- | --- |
| Min Track Width | 6 mil |
| Min Clearance | 6 mil |
| Via Drill | ≥ 0.3 mm |
| Via Diameter | ≥ 0.6 mm |
| Annular Ring | ≥ 0.15 mm |
| Copper Thickness | 1 oz (2 oz recommended) |
| Board Thickness | 1.6 mm |

Important Point:
DRC must pass with zero errors before fabrication.

## 5. 2D / 3D PCB Views & Final Checks

### 5.1 2D PCB View

- Verify silkscreen readability

- Check reference designators

- Confirm polarity markings

### 5.2 3D PCB View

- Validate connector orientation

- Ensure no mechanical interference

- Check component heights

### 5.3 Final Design Checks

- ERC (Electrical Rule Check) passed

- DRC passed

- Power net continuity verified

- No floating pins

- Correct net naming

- Thermal reliefs on large copper areas

## 6. Production Files (Fabrication & Assembly)

### 6.1 Fabrication Files

- Gerber files (Top, Bottom, Mask, Silkscreen)

- Drill files (Excellon)

- Board outline

- Fabrication drawing (PDF)

### 6.2 Assembly Files

- Pick & Place file

- Assembly drawing (Top/Bottom)

- BOM with manufacturer part numbers

## 7. Bill of Materials (BOM – Example)

| Item | Component | Value | Package | Importance |
| --- | --- | --- | --- | --- |
| MCU | ATmega328P | — | TQFP-32 | Control |
| Motor Driver | TB6612FNG | — | SOP-16 | Drive |
| Buck Regulator | LM2596 | — | D2PAK | Power |
| RF Module | nRF24L01 | — | Module | Communication |
| Capacitors | Ceramic | 0.1µF | 0603 | Decoupling |
| Electrolytic | Capacitor | 470µF | TH | Filtering |
| Resistors | Pull-ups | 10kΩ | 0603 | Stability |
| Battery Conn | XT30 | — | TH | Power |
| Motor Conn | Screw Terminal | — | TH | Output |

## 8. Important Design Considerations (Summary)

- Separate motor and logic grounds

- Use wide traces for high current

- Maintain RF clearance

- Add power protection

- Follow manufacturer footprint guidelines

- Always run ERC & DRC before fabrication